ECA5403-ELECTROMAGNETIC FILEDS AND WAVES

C02-2

EXPERIMENT 7

Aim:

To determine and plot the capacitance of an isolated sphere and a concentric spherical capacitor for different radii and dielectric constants using MATLAB.

Test Case (i) – Capacitance of an Isolated Sphere

Determine the capacitance for a specified radius and relative permittivity.

MATLAB Program:

clc;

clear;

epsilon0 = 8.854e-12;

a = input('Enter radius of isolated sphere (m): ');

er = input('Enter relative permittivity: ');

epsilon = epsilon0*er;

C = 4*pi*epsilon*a;

disp('Capacitance of isolated sphere (F) =');

disp(C);

OUTPUT:

Test Case (ii) – Variation of Isolated-Sphere Capacitance with Radius

clc;

clear;

epsilon0 = 8.854e-12;

er = 1;

a = linspace(0.1,2,100);

C = 4*pi*epsilon0*er.*a;

plot(a,C,'LineWidth',2); grid on;

xlabel('Radius a (m)'); ylabel('Capacitance C (F)');

title('Capacitance of an Isolated Sphere vs Radius');

OUTPUT:

Test Case (iii) – Capacitance of a Concentric Spherical Capacitor

Determine the capacitance for specified inner radius a, outer radius b and relative permittivity εr.

clc;

clear;

epsilon0 = 8.854e-12;

a = input('Enter inner radius a (m): ');

b = input('Enter outer radius b (m): ');

er = input('Enter relative permittivity: ');

epsilon = epsilon0*er;

C = 4*pi*epsilon*a*b/(b-a);

disp('Capacitance of concentric spherical capacitor (F) =');

disp(C);

OUTPUT:

Test Case (iv) – Effect of Outer Radius and Relative Permittivity

clc;

clear;

epsilon0 = 8.854e-12;

a = 0.05;

er = 2.5;

b = linspace(0.06,0.20,100);

C1 = 4*pi*epsilon0*er.*a.*b./(b-a);

figure; plot(b,C1,'LineWidth',2);

grid on;

xlabel('Outer radius b (m)');

ylabel('Capacitance C (F)');

title('Concentric Sphere: Capacitance vs Outer Radius');

er = linspace(1,6,100);

b = 0.10;

C2 = 4*pi*epsilon0.*er.*a.*b./(b-a);

figure; plot(er,C2,'LineWidth',2);

grid on;

xlabel('Relative permittivity er');

ylabel('Capacitance C (F)');

title('Concentric Sphere: Capacitance vs Relative Permittivity');

OUTPUT:

| Objective | Assessment Criterion | Excellent – 4 | Good – 3 | Satisfactory – 2 | Needs Improvement – 1 | Weighting | Marks given | Marks obtained |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Define spherical geometry and dielectric | Define spherical geometry and dielectric | Correctly identifies isolated-sphere radius or inner/outer radii and medium permittivity. | Geometry is correct with minor omission. | Some dimensions or material properties are unclear. | Uses invalid geometry or material inputs. | 25 | 2.5 |  |
| Implement spherical capacitance equations | Implement spherical capacitance equations | Applies the appropriate isolated/concentric sphere formulas correctly. | Formulas are mostly correct with minor errors. | Partial calculations require guidance. | Uses incorrect equations. | 25 | 2.5 |  |
| Analyze radius and permittivity effects | Analyze radius and permittivity effects | Compares cases and validates the expected capacitance trends against theory. | Analysis is mostly correct with minor omissions. | Comparisons or validation are incomplete. | No valid trend analysis is shown. | 30 | 3.0 |  |
| Present results and conclusions | Present results and conclusions | Reports geometry, units, calculations and conclusions clearly. | Report is clear with minor gaps. | Report lacks some key information. | Results are unclear or missing. | 20 | 2.0 |  |
| TOTAL |  |  |  |  |  | 100 | 10 |  |